LAB NOTE

Subject: Digital Design Principles

Topic: Seven Segment Display

Student: Minh Quan Tran

May 31th, 2024

Table of Contents

1.	Objectives	4

2.	Design	6

2.1	Requirement	6

2.2	Solution	6

3.	VHDL and Verilog	6

3.1	VHDL code	6

3.2	Verilog code	9

4.	Pin Planner	14

4.1	Input and Output	14

5.	Result	16

TABLE OF FIGURES

Figure 4-1: SCEMA5F31C6N board	14

Figure 4-2: SW0 and SW1 Pin No	15

Figure 4-3: LEDR0's Pin No	15

Figure 4-4: HEX's Pin No	15

Figure 5-1: When all 6 levels are available.	16

Figure 5-2: When 1 level is full	16

Figure 5-3: When all levels are full	17

# Objectives

- Design a parking indicator using Quartus software.

- Write a VHDL and Verilog code from the design.

- Push the code and run to the SCEMA5F31C6N board.

# Design

## Requirement

- Use VHDL to implement a binary to 7 Segment decoder.

- Introduction to the VHDL Case statement.

- Above and beyond:  Create a calculator that can perform basic addition up to 2 digit.

## Solution

To design this system, first need to identify input and output:

- Input will be from SW0 to SW3.

- Output will be the 7 LED from Seven Segment Led.

Then identify what the output will be from the 4 Switches, because the LED is active high so the code for 7 segment code will be:

| No | Input | Output |
| --- | --- | --- |
| 0 | 0000 | 100 0000 |
| 1 | 0001 | 111 1001 |
| 2 | 0010 | 010 0100 |
| 3 | 0011 | 011 0000 |
| 4 | 0100 | 001 1001 |
| 5 | 0101 | 001 0010 |
| 6 | 0110 | 000 0010 |
| 7 | 0111 | 111 1000 |
| 8 | 1000 | 000 0000 |
| 9 | 1001 | 001 0000 |
| 10 | 1010 | 000 1000 |
| 11 | 1011 | 000 0011 |
| 12 | 1100 | 100 0110 |
| 13 | 1101 | 010 0001 |
| 14 | 1110 | 000 0110 |
| 15 | 1111 | 000 1110 |

# VHDL and Verilog

## VHDL code

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

-- Declaring input and output

entity MTran_Lab4_VHDL_7SegmentLed is

port

(

SW :  in  std_logic_vector (9 downto 0);

BT :  in  std_logic_vector (3 downto 0);

LED:  out std_logic_vector (7 downto 0);

HEX0: out std_logic_vector (6 downto 0);

HEX1: out std_logic_vector (6 downto 0);

HEX2: out std_logic_vector (6 downto 0);

HEX3: out std_logic_vector (6 downto 0)

);

end MTran_Lab4_VHDL_7SegmentLed;

-- Describing the relationship between output and input

architecture behavioral of MTran_Lab4_VHDL_7SegmentLed is

shared variable num1_flag, num2_flag, add_flag, err_num1, err_num2: Boolean := false;

shared variable hundred, tenth, unit: integer := 0;

shared variable err, digit0, digit1, digit2, digit3, num1, num2, result: integer := 0;

-- Function declaration

-- Assignment

function SevenSegmentDisplay(Switch : std_logic_vector (3 downto 0)) return std_logic_vector is

begin

case (Switch) is

when "0000" => return "1000000";

when "0001" => return "1111001";

when "0010" => return "0100100";

when "0011" => return "0110000";

when "0100" => return "0011001";

when "0101" => return "0010010";

when "0110" => return "0000010";

when "0111" => return "1111000";

when "1000" => return "0000000";

when "1001" => return "0010000";

when "1010" => return "0001000";

when "1011" => return "0000011";

when "1100" => return "1000110";

when "1101" => return "0100001";

when "1110" => return "0000110";

when "1111" => return "0001110";

when others => return "1111111";

end case;

end SevenSegmentDisplay;

-- Displaying Digit

function DigitDisplay(Switch : std_logic_vector (3 downto 0);dig : integer) return std_logic_vector is

begin

if (dig <= 9) then return SevenSegmentDisplay(Switch);

else

return SevenSegmentDisplay("1111");

end if;

end DigitDisplay;

-- Check Digit

function DigitCheck(dig0 : integer; dig1 : integer) return Boolean is

begin

if (dig0 > 9 or dig1 > 9) then

return true;

else return false;

end if;

end DigitCheck;

-- End of Function Declaration

-- Main code

begin

-- Check SW

Control_Number: process(SW,BT(3))

begin

-- Reset

err := 0;

if (BT(3) = '0') then

HEX3 <= "1111111";

HEX2 <= "1111111";

HEX1 <= "1111111";

HEX0 <= "1111111";

err_num1 := false;

err_num2 := false;

end if;

-- SW9 to change different assignment

if (SW(9) = '0') then

HEX3 <= "1111111";

HEX2 <= "1111111";

HEX1 <= "1111111";

HEX0 <= SevenSegmentDisplay(SW(3 downto 0));

else

-- Displaying number 1 and number 2

if (add_flag = false) then

if (num1_flag = false) then

digit0 := to_integer(unsigned(SW(3 downto 0)));

HEX0 <= DigitDisplay(SW(3 downto 0),digit0);

digit1 := to_integer(unsigned(SW(7 downto 4)));

HEX1 <= DigitDisplay(SW(7 downto 4),digit1);

-- Check num1

err_num1 := DigitCheck(digit0, digit1);

-- Turn off HEX2 and

HEX2 <= "1111111";

HEX3 <= "1111111";

elsif (num2_flag = false) then

digit2 := to_integer(unsigned(SW(3 downto 0)));

HEX2 <= DigitDisplay(SW(3 downto 0),digit2);

digit3 := to_integer(unsigned(SW(7 downto 4)));

HEX3 <= DigitDisplay(SW(7 downto 4),digit3);

-- Check num2

err_num2 := DigitCheck(digit3, digit2);

-- Turn off HEX0 and

HEX1 <= "1111111";

HEX0 <= "1111111";

end if;

else

-- Add up 2 number then display

result := num1 + num2;

hundred := result / 100;

tenth   := (result / 10) mod 10;

unit    := result mod 10;

if (result >= 100) then

HEX3 <= "1111111";

HEX2 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(hundred,4)));

HEX1 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(tenth,4)));

HEX0 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(unit,4)));

elsif result >= 10 then

HEX3 <= "1111111";

HEX2 <= "1111111";

HEX1 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(tenth,4)));

HEX0 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(unit,4)));

else

HEX3 <= "1111111";

HEX2 <= "1111111";

HEX1 <= "1111111";

HEX0 <= SevenSegmentDisplay(std_logic_vector(to_unsigned(unit,4)));

end if;

-- Check if error

if (err_num1 or err_num2) then

HEX3 <= "1111111";

HEX2 <= "0000110"; --E

HEX1 <= "0101111"; --r

HEX0 <= "0101111"; --r

end if;

end if;

end if;

end process Control_Number;

-- BT0 to confirm number

Check_BT0: process (BT(0),BT(3))

begin

if (BT(3) = '0') then

LED (3 downto 0) <= "0000";

num1_flag := false;

end if;

if (BT(0) = '0') then

if (num1_flag = false) then

num1_flag := true;

if not err_num1 then

num1 := digit1 * 10 + digit0;

else

LED (3 downto 0) <= "1111";

end if;

end if;

end if;

end process Check_BT0;

-- BT1 to confirm number 2

Check_BT1: process (BT(1),BT(3))

begin

if (BT(3) = '0') then

LED (7 downto 4) <= "0000";

num2_flag := false;

end if;

if (BT(1) = '0') then

if (num2_flag = false) then

num2_flag := true;

if not err_num2 then

num2 := digit3 * 10 + digit2;

else

LED (7 downto 4) <= "1111";

end if;

end if;

end if;

end process Check_BT1;

-- Check BT2

Calculate_BT2: process (BT(2), BT(3))

begin

if BT(3) = '0' then

add_flag := false;

end if;

if BT(2) = '0' then

add_flag := true;

end if;

end process Calculate_BT2;

end behavioral;

## Verilog code

module MTran_Lab4_Verilog_7SegmentLed (

input [9:0] SW,

input [3:0] BT,

output reg [7:0] LED,

output reg [6:0] HEX0,

output reg [6:0] HEX1,

output reg [6:0] HEX2,

output reg [6:0] HEX3

);

// Variable declaration

reg add_flag, num1_flag, num2_flag;

integer hundred, tenth, unit;

integer err, err_num1, err_num2, digit0, digit1, digit2, digit3, num1, num2, result;

initial

begin

add_flag = 0;

num1_flag = 0;

num2_flag = 0;

hundred = 0;

tenth = 0;

unit = 0;

err = 0;

err_num1 = 0;

err_num2 = 0;

digit0 = 0;

digit1 = 0;

digit2 = 0;

digit3 = 0;

num1 = 0;

num2 = 0;

result = 0;

end

// Function Declaration

// Displaying 7 segment function

function [6:0] SevenSegmentDisplay;

input [3:0] Switch;

begin

case (Switch)

4'b0000: SevenSegmentDisplay = 7'b1000000;

4'b0001: SevenSegmentDisplay = 7'b1111001;

4'b0010: SevenSegmentDisplay = 7'b0100100;

4'b0011: SevenSegmentDisplay = 7'b0110000;

4'b0100: SevenSegmentDisplay = 7'b0011001;

4'b0101: SevenSegmentDisplay = 7'b0010010;

4'b0110: SevenSegmentDisplay = 7'b0000010;

4'b0111: SevenSegmentDisplay = 7'b1111000;

4'b1000: SevenSegmentDisplay = 7'b0000000;

4'b1001: SevenSegmentDisplay = 7'b0010000;

4'b1010: SevenSegmentDisplay = 7'b0001000;

4'b1011: SevenSegmentDisplay = 7'b0000011;

4'b1100: SevenSegmentDisplay = 7'b1000110;

4'b1101: SevenSegmentDisplay = 7'b0100001;

4'b1110: SevenSegmentDisplay = 7'b0000110;

4'b1111: SevenSegmentDisplay = 7'b0001110;

default: SevenSegmentDisplay = 7'b1111111;

endcase

end

endfunction

// Function to display a digit on 7-segment display

function [6:0] DigitDisplay;

input [3:0] Switch;

input integer dig;

begin

if (dig <= 9)

begin

DigitDisplay = SevenSegmentDisplay(Switch);

end

else

begin

err = err + 1;

DigitDisplay = SevenSegmentDisplay(4'b1111);

end

end

endfunction

// Main code

//

always @(SW)

begin

// Reset

err = 0;

// SW9 to change different assignment

if (SW[9] == 1'b0)

begin

HEX3 = 7'b1111111;

HEX2 = 7'b1111111;

HEX1 = 7'b1111111;

HEX0 = SevenSegmentDisplay(SW[3:0]);

end

else

begin

// Displaying number 1 and number 2

if (!add_flag)

begin

if (!num1_flag)

begin

digit0 = SW[3:0];

HEX0 = DigitDisplay(SW[3:0], digit0);

digit1 = SW[7:4];

HEX1 = DigitDisplay(SW[7:4], digit1);

// Turn off HEX2 and HEX3

HEX2 = 7'b1111111;

HEX3 = 7'b1111111;

end

else

begin

digit2 = SW[3:0];

HEX2 = DigitDisplay(SW[3:0], digit2);

digit3 = SW[7:4];

HEX3 = DigitDisplay(SW[7:4], digit3);

// Turn off HEX0 and HEX1

HEX1 = 7'b1111111;

HEX0 = 7'b1111111;

end

end

else

// Add up 2 number then display

begin

result = num1 + num2;

hundred = result / 100;

tenth   = (result / 10) % 10;

unit    = result % 10;

if (result >= 100)

begin

HEX3 = 7'b1111111;

HEX2 = SevenSegmentDisplay(hundred);

HEX1 = SevenSegmentDisplay(tenth);

HEX0 = SevenSegmentDisplay(unit);

end

else if (result >= 10)

begin

HEX3 = 7'b1111111;

HEX2 = 7'b1111111;

HEX1 = SevenSegmentDisplay(tenth);

HEX0 = SevenSegmentDisplay(unit);

end

else

begin

HEX3 = 7'b1111111;

HEX2 = 7'b1111111;

HEX1 = 7'b1111111;

HEX0 = SevenSegmentDisplay(unit);

end

// Check if error

if (err_num1 || err_num2)

begin

HEX3 = 7'b1111111;

HEX2 = 7'b0000110; //E

HEX1 = 7'b0101111; //r

HEX0 = 7'b0101111; //r

end

end

end

end

// Check BT0

always@ (negedge(BT[0]) or negedge(BT[3]))

begin

// Reset

if (!BT[3])

begin

num1_flag = 0;

err_num1 = 0;

LED[3:0] = 4'b0000;

end

// Assigning num1

if (~BT[0])

begin

num1_flag = 1;

if (err == 0)

begin

num1 = digit1 * 10 + digit0;

end

else

begin

LED[3:0] = 4'b1111;

err_num1 = 1;

end

end

end

// Check BT1

always@ (negedge(BT[1]) or negedge(BT[3]))

begin

// Reset

if (!BT[3])

begin

num2_flag = 0;

err_num2 = 0;

LED[7:4] = 4'b0000;

end

// Assigning num2

if (!BT[1])

begin

num2_flag = 1;

if (err == 0)

begin

num2 = digit3 * 10 + digit2;

end

else

begin

LED[7:4] = 4'b1111;

err_num2 = 1;

end

end

end

always@ (negedge(BT[2]) or negedge(BT[3]))

begin

if (!BT[3])

begin

add_flag = 0;

end

else

begin

add_flag = 1;

end

end

endmodule

# Pin Planner

## Input and Output

Figure 4-1: SCEMA5F31C6N board

Assigning:

BT[0]:         KEY0

BT[1]:         KEY1

BT[2]:         KEY2

SW[0]:         SW0

SW[1]:         SW1

SW[2]:         SW2

SW[3]:         SW3

SW[4]:         SW4

SW[5]:         SW5

SW[6]:         SW6

SW[7]:         SW7

SW[8]:         SW8

SW[9]:         SW9

LED[0]:       LEDR0

LED[1]:       LEDR1

LED[2]:       LEDR2

LED[3]:       LEDR3

LED[4]:       LEDR4

LED[5]:       LEDR5

LED[6]:       LEDR6

LED[7]:       LEDR7

| HEX0[0]: HEX0[0] HEX0[1]: HEX0[1] HEX0[2]: HEX0[2] HEX0[3]: HEX0[3] HEX0[4]: HEX0[4] HEX0[5]: HEX0[5] HEX0[6]: HEX0[6] | HEX1[0]: HEX1[0] HEX1[1]: HEX1[1] HEX1[2]: HEX1[2] HEX1[3]: HEX1[3] HEX1[4]: HEX1[4] HEX1[5]: HEX1[5] HEX1[6]: HEX1[6] | HEX2[0]: HEX2[0] HEX2[1]: HEX2[1] HEX2[2]: HEX2[2] HEX2[3]: HEX2[3] HEX2[4]: HEX2[4] HEX2[5]: HEX2[5] HEX2[6]: HEX2[6] | HEX3[0]: HEX3[0] HEX3[1]: HEX3[1] HEX3[2]: HEX3[2] HEX3[3]: HEX3[3] HEX3[4]: HEX3[4] HEX3[5]: HEX3[5] HEX3[6]: HEX3[6] |
| --- | --- | --- | --- |

From the DE1_SoC_User_Manual,

Figure 4-2: SW0 and SW1 Pin No

Figure 4-3: LEDR0's Pin No

Figure 4-4: HEX's Pin No

# Result

Figure 5-1: When all 6 levels are available.

SW[3:0] all equal to 0 in which displaying 0.

Figure 5-2: When 1 level is full

SW3 = 1, SW2 = 0, SW1 = 0, SW0 = 1 which should be 9 and the 7 segment LED displaying 9.

Figure 5-3: When all levels are full

SW3 = 1, SW2 = 1, SW1 = 1, SW0 = 0 which should be 14 and the 7 segment LED displaying ‘E’.

Calculator:

Figure 5-4: Choosing number 1

Input are SW[7:0] to choose number 1. SW[7:4] controlled HEX1 and SW[3:0] controlled HEX0.

Input BT0 is used to confirm the first number.

Figure 5-5: Choosing number 2

Also input are SW[7:0] to choose number 2. SW[7:4] controlled HEX3 and SW[3:0] controlled HEX2.

Input BT1 is used to confirm the number.

Figure 5-6: Result for adding them up

After confirming both number, press button 2 to add both number and HEX[2:0] will display the result.

REFERENCES