LAB NOTE

Subject: Digital Design Principles

Topic: Octal Decoder

Student: Minh Quan Tran

Jun 6th, 2024

Table of Contents

1.	Objectives	4

2.	Theory and Design	6

2.1	Theory	6

2.2	Requirement	6

2.3	Solution	6

3.	VHDL and Verilog	6

3.1	VHDL code	6

3.2	Verilog code	7

4.	Pin Planner	10

4.1	Input and Output	10

5.	Result	12

TABLE OF FIGURES

Figure 2-1: Octal Decoder's Truth Table	6

Figure 4-1: SCEMA5F31C6N board	10

Figure 4-2: SW0 and SW1 Pin No	11

Figure 4-3: LEDR0's Pin No	11

Figure 4-4: HEX's Pin No	11

Figure 5-1: When input = ‘000’.	12

Figure 5-2: When input = ‘111’	12

Figure 5-3: When input ‘101’	13

Figure 5-4: Input ‘101’ with Complement mode	13

Figure 5-5: Input ‘101’ with Multiline mode	14

Figure 5-6: Result for adding them up	14

# Objectives

- Design an Octal Decoder using Quartus software.

- Write a VHDL and Verilog code from the design.

- Push the code and run to the SCEMA5F31C6N board.

# Theory and Design

## Theory

Decoder circuit is a combinational logic circuit used to decode binary input. As an example consider the octal decoder depicted in fig.(1). It has three inputs; x2, x1, x0 and eight outputs y0 to y7. . Only one of the output lines is asserted at any time. The valuation of the binary number of the input lines: (x2, x1, x0) determines which output line is activated. Other names of the octal decoder are 3-to-8 decoder and 1-out-of-8 decoder. The truth table in fig. (2) demonstrates the relation between the outputs and the inputs of the decoder.

In any decoder, the index of the asserted output is always equal to the valuation of the binary number of the input lines. Applying 100 at the input lines (x2, x1, x0) will assert output y4 and leave all other outputs are de-asserted.

Number of input lines N of the decoder is calculated based on the number of outputs M:

N= log2 M

Other decoder circuits are the 2-to-4 decoder, decimal decoder, hexadecimal decode.

## Requirement

- Use VHDL and Verilog to implement an Octal Decoder and display to LED [0:7].

- Introduction to the VHDL Case statement.

- Above and beyond:  Adding 3 more modes: Complement Mode, Multiline Mode, Complement Multiline Mode.

## Solution

To design this system, first need to identify input and output:

- Input will be from SW0 to SW2.

- Output will be the 7 LED.

Then identify what the output will be from the 3 Switches,

Figure 2-1: Octal Decoder's Truth Table

# VHDL and Verilog

## VHDL code

library ieee;

use ieee.std_logic_1164.all;

-- Declaring input and output

entity MTran_Lab5_VHDL_OctalDecoder is

port

(

EN   : in  std_logic;

SW   : in  std_logic_vector (2 downto 0);

mode : in  std_logic_vector (1 downto 0);

LED  : out std_logic_vector (7 downto 0);

HEX  : out std_logic_vector (6 downto 0)

);

end MTran_Lab5_VHDL_OctalDecoder;

-- Describing the relationship between output and input

architecture behavioral of MTran_Lab5_VHDL_OctalDecoder is

-- Variable Declaring

signal LED_output : std_logic_vector (7 downto 0) := "00000000";

-- Function's Declaration

-- Function for Octal Decoder

function Decoder3to8_NormalMode(Switch: in std_logic_vector (2 downto 0)) return std_logic_vector is

begin

case Switch is

when "000" => return "00000001";

when "001" => return "00000010";

when "010" => return "00000100";

when "011" => return "00001000";

when "100" => return "00010000";

when "101" => return "00100000";

when "110" => return "01000000";

when "111" => return "10000000";

end case;

end Decoder3to8_NormalMode;

function Decoder3to8_MultilineMode(Switch: in std_logic_vector (2 downto 0)) return std_logic_vector is

begin

case Switch is

when "000" => return "00000001";

when "001" => return "00000011";

when "010" => return "00000111";

when "011" => return "00001111";

when "100" => return "00011111";

when "101" => return "00111111";

when "110" => return "01111111";

when "111" => return "11111111";

end case;

end Decoder3to8_MultilineMode;

-- Seven Segment Display

function SevenSegmentDisplay(Switch : std_logic_vector (2 downto 0)) return std_logic_vector is

begin

case Switch is

when "000" => return "1000000";

when "001" => return "1111001";

when "010" => return "0100100";

when "011" => return "0110000";

when "100" => return "0011001";

when "101" => return "0010010";

when "110" => return "0000010";

when "111" => return "1111000";

end case;

end SevenSegmentDisplay;

begin

process(SW, mode)

begin

if EN = '1' then

case mode is

-- Normal Mode

when "00" =>

LED <= Decoder3to8_NormalMode(SW);

HEX <= SevenSegmentDisplay(SW);

-- Complement Mode

when "01" =>

LED_output <= Decoder3to8_NormalMode(SW);

HEX <= SevenSegmentDisplay(SW);

LED <= not LED_output;

-- Multiline Mode

when "10" =>

LED <= Decoder3to8_MultilineMode(SW);

HEX <= SevenSegmentDisplay(SW);

-- Complement Multiline Mode

when "11" =>

LED_output <= Decoder3to8_MultilineMode(SW);

HEX <= SevenSegmentDisplay(SW);

LED <= not LED_output;

end case;

else

LED <= "00000000";

HEX <= "1111111";

end if;

end process;

end behavioral;

## Verilog code

module MTran_Lab5_Verilog_OctalDecoder (

input EN,

input [2:0] SW,

input [1:0] mode,

output reg [7:0] LED,

output reg [6:0] HEX

);

// Variable Declaring

reg [7:0] LED_output;

// Function's Declaration

// Function for Octal Decoder

function [7:0] Decoder3to8_NormalMode;

input [2:0] Switch;

begin

case (Switch)

3'd0: Decoder3to8_NormalMode = 8'b00000001;

3'd1: Decoder3to8_NormalMode = 8'b00000010;

3'd2: Decoder3to8_NormalMode = 8'b00000100;

3'd3: Decoder3to8_NormalMode = 8'b00001000;

3'd4: Decoder3to8_NormalMode = 8'b00010000;

3'd5: Decoder3to8_NormalMode = 8'b00100000;

3'd6: Decoder3to8_NormalMode = 8'b01000000;

3'd7: Decoder3to8_NormalMode = 8'b10000000;

default: Decoder3to8_NormalMode = 8'b00000000;

endcase

end

endfunction

function [7:0] Decoder3to8_MultilineMode;

input [2:0] Switch;

begin

case (Switch)

3'd0: Decoder3to8_MultilineMode = 8'b00000001;

3'd1: Decoder3to8_MultilineMode = 8'b00000011;

3'd2: Decoder3to8_MultilineMode = 8'b00000111;

3'd3: Decoder3to8_MultilineMode = 8'b00001111;

3'd4: Decoder3to8_MultilineMode = 8'b00011111;

3'd5: Decoder3to8_MultilineMode = 8'b00111111;

3'd6: Decoder3to8_MultilineMode = 8'b01111111;

3'd7: Decoder3to8_MultilineMode = 8'b11111111;

default: Decoder3to8_MultilineMode = 8'b00000000;

endcase

end

endfunction

// Seven Segment Display

function [6:0] SevenSegmentDisplay;

input [2:0] Switch;

begin

case (Switch)

3'd0: SevenSegmentDisplay = 7'b1000000;

3'd1: SevenSegmentDisplay = 7'b1111001;

3'd2: SevenSegmentDisplay = 7'b0100100;

3'd3: SevenSegmentDisplay = 7'b0110000;

3'd4: SevenSegmentDisplay = 7'b0011001;

3'd5: SevenSegmentDisplay = 7'b0010010;

3'd6: SevenSegmentDisplay = 7'b0000010;

3'd7: SevenSegmentDisplay = 7'b1111000;

default: SevenSegmentDisplay = 7'b1111111;

endcase

end

endfunction

// Main code

always @(SW,EN,mode)

begin

if (EN)

begin

case (mode)

// Normal Mode

2'b00:

begin

LED = Decoder3to8_NormalMode(SW);

HEX = SevenSegmentDisplay(SW);

end

// Complement Mode

2'b01:

begin

LED_output = Decoder3to8_NormalMode(SW);

HEX = SevenSegmentDisplay(SW);

LED = ~LED_output;

end

// Multiline Mode

2'b10:

begin

LED = Decoder3to8_MultilineMode(SW);

HEX = SevenSegmentDisplay(SW);

end

// Complement Multiline Mode

2'b11:

begin

LED_output = Decoder3to8_MultilineMode(SW);

HEX = SevenSegmentDisplay(SW);

LED = ~LED_output;

end

endcase

end

else

begin

LED = 8'b00000000;

HEX = 7'b1111111;

end

end

endmodule

# Pin Planner

## Input and Output

Figure 4-1: SCEMA5F31C6N board

Assigning:

SW[0]:         SW0

SW[1]:         SW1

SW[2]:         SW2

Mode[0]:         SW7

Mode[1]:         SW8

EN:         	SW9

LED[0]:       LEDR0

LED[1]:       LEDR1

LED[2]:       LEDR2

LED[3]:       LEDR3

LED[4]:       LEDR4

LED[5]:       LEDR5

LED[6]:       LEDR6

LED[7]:       LEDR7

HEX0[0]:       HEX0[0]

HEX0[1]:       HEX0[1]

HEX0[2]:       HEX0[2]

HEX0[3]:       HEX0[3]

HEX0[4]:       HEX0[4]

HEX0[5]:       HEX0[5]

HEX0[6]:       HEX0[6]

From the DE1_SoC_User_Manual,

Figure 4-2: SW0 and SW1 Pin No

Figure 4-3: LEDR0's Pin No

Figure 4-4: HEX's Pin No

# Result

Figure 5-1: When input = ‘000’.

SW2 = 0, SW1 = 0, SW0 = 0 which should be 0, LED0 asserted and the 7 segment LED displaying 0.

Figure 5-2: When input = ‘111’

SW2 = 1, SW1 = 1, SW0 = 1 which should be 7, LED7 asserted and the 7 segment LED displaying 7.

Figure 5-3: When input ‘101’

SW2 = 1, SW1 = 0, SW0 = 1 which should be 5, LED5 asserted and the 7 segment LED displaying 5.

Different mode:

- Complement mode:

Figure 5-4: Input ‘101’ with Complement mode

In this mode, rather than LED5 assert, all other LED except LED5 asserted. To activate this mode SW [8:7] = ‘01’.

- Multiline Mode:

Figure 5-5: Input ‘101’ with Multiline mode

In this mode, when input is ‘101’ which is 5, LED0 to LED5 lit up. To activate this mode SW [8:7] = ‘10’.

- Complement Multiline Mode:

Figure 5-6: Result for adding them up

In this mode, it take the output of Multiline Mode and complemented it. To activate this mode SW [8:7] = ‘11’

REFERENCES