LAB NOTE

Subject: Digital Design Principles

Topic: Half Adder

Student: Minh Quan Tran

July 5th, 2024

Table of Contents

1.	Objectives	4

2.	Theory and Design	6

2.1	Theory	6

2.2	Requirement	6

2.3	Solution	6

3.	VHDL and Verilog	6

3.1	VHDL code	6

3.2	Verilog code	8

4.	Pin Planner	11

4.1	Input and Output	11

5.	Result	13

5.1	Half Adder	13

5.2	Half Subtractor	13

TABLE OF FIGURES

Figure 2-1: Half Adder Truth Table	6

Figure 2-2: Half Subtractor's truth table	6

Figure 4-1: SCEMA5F31C6N board	11

Figure 4-2: SW0 and SW1 Pin No	12

Figure 4-3: LEDR0's Pin No	12

Figure 4-4: HEX's Pin No	12

Figure 5-1: Output for Half Adder.	13

Figure 5-2: Output for Half Subtractor	13

# Objectives

- Design an Half Adder using Quartus software.

- Write a VHDL and Verilog code from the design.

- Push the code and run to the SCEMA5F31C6N board.

# Theory and Design

## Theory

Half adder is used for adding a basic function of digital circuits.

- It is useful in many data processing applications.

- This circuit implements the following:

- 0 + 0 = 00

- 0 + 1 = 01

- 1 + 0 = 01

- 1 + 1 = 10

## Requirement

- Use VHDL and Verilog to implement half adder and display to LED [0:7].

- Introduction to the VHDL Case statement.

- Above and beyond:  Adding 1 more modes: Half Subtractor

## Solution

To design this system, first need to identify input and output:

- Input A will be from SW0 to SW3.

- Input B will be from SW4 to SW7.

- Output will be the 7 LED.

Then identify what the output will be using the below truth table,

Figure 2-1: Half Adder Truth Table

Figure 2-2: Half Subtractor's truth table

# VHDL and Verilog

## VHDL code

library ieee;

use ieee.std_logic_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity MTran_Lab6_VHDL_Adder is

port

(

mode  : in   std_logic_vector (9 downto 8);

A     : in   std_logic_vector (3 downto 0);

B     : in   std_logic_vector (7 downto 4);

LED   : out  std_logic_vector (7 downto 0);

HEX1  : out  std_logic_vector (6 downto 0);

HEX0  : out  std_logic_vector (6 downto 0)

);

end MTran_Lab6_VHDL_Adder;

architecture Behavioral of MTran_Lab6_VHDL_Adder is

-------------------------- Component Declaration---------------------------------

--Half Adder

component MTran_Lab6_VHDL_HalfAdder is

port

(

A     : in   std_logic_vector (3 downto 0);

B     : in   std_logic_vector (7 downto 4);

Sum   : out  std_logic_vector (3 downto 0);

Carry : out  std_logic_vector (3 downto 0)

);

end component;

--Half Subtractor

component MTran_Lab6_VHDL_HalfSubtractor is

port

(

A            : in   std_logic_vector (3 downto 0);

B            : in   std_logic_vector (7 downto 4);

Difference   : out  std_logic_vector (3 downto 0);

Borrow       : out  std_logic_vector (3 downto 0)

);

end component;

--Full Adder

component MTran_Lab6_VHDL_FullAdder is

port

(

A            : in   std_logic_vector (3 downto 0);

B            : in   std_logic_vector (3 downto 0);

Cin          : in   std_logic;

Sum          : out  std_logic_vector (3 downto 0);

CarryOut     : out  std_logic

);

end component;

-------------------------- End of Component Declaration--------------------------

-------------------------- Function Declaration----------------------------------

-- Function to calculate Sum of Half Adder

-- Seven Segment Display

function SevenSegmentDisplay(Number : integer) return std_logic_vector is

begin

case (Number) is

when 0 => return "1000000";

when 1 => return "1111001";

when 2 => return "0100100";

when 3 => return "0110000";

when 4 => return "0011001";

when 5 => return "0010010";

when 6 => return "0000010";

when 7 => return "1111000";

when 8 => return "0000000";

when 9 => return "0010000";

when others => return "1111111";

end case;

end SevenSegmentDisplay;

--------------------------End of Function Declaration--------------------------

--------------------------Variable and Signal Declaration----------------------

signal Sum_HA : std_logic_vector(3 downto 0);

signal Carry_HA : std_logic_vector(3 downto 0);

signal Difference_HS: std_logic_vector(3 downto 0);

signal Borrow_HS: std_logic_vector(3 downto 0);

signal Sum_FA : std_logic_vector(3 downto 0);

signal Carry_FA : std_logic;

signal Cin_FA: std_logic;

signal TotalSum : integer;

signal tenth    : integer;

signal unit     : integer;

--------------------------End of Variable and Signal Declaration---------------

begin

-- Main code

-- Component

HalfAdder: MTran_Lab6_VHDL_HalfAdder

port map(

A => A,

B => B,

Sum => Sum_HA,

Carry => Carry_HA

);

HalfSubtractor: MTran_Lab6_VHDL_HalfSubtractor

port map(

A => A,

B => B,

Difference => Difference_HS,

Borrow => Borrow_HS

);

FullAdder: MTran_Lab6_VHDL_FullAdder

port map(

A => A,

B => B,

Cin => Cin_FA,

Sum => Sum_FA,

CarryOut => Carry_FA

);

process(mode, A, B)

begin

-- Reset Seven Segment Display and LED

HEX1 <= "1111111";

HEX0 <= "1111111";

LED  <= "00000000";

-- Change mode

if mode = "00" then                 -- Half Adder mode

LED(3 downto 0) <= Sum_HA;

LED(7 downto 4) <= Carry_HA;

elsif mode = "01" then        -- Half Subtractor mode

LED(3 downto 0) <= Difference_HS;

LED(7 downto 4) <= Borrow_HS;

elsif mode = "10" then        -- Full Adder with Cin = 0  mode

Cin_FA <= '0';

LED(3 downto 0) <= Sum_FA;

LED(4) <= Carry_FA;

TotalSum <= to_integer(unsigned(Carry_FA & Sum_FA));

tenth <= TotalSum / 10;

unit  <= TotalSum mod 10;

HEX1 <= SevenSegmentDisplay(tenth);

HEX0 <= SevenSegmentDisplay(unit);

elsif mode = "11" then        -- Full Adder with Cin = 1  mode

Cin_FA <= '1';

LED(3 downto 0) <= Sum_FA;

LED(4) <= Carry_FA;

TotalSum <= to_integer(unsigned(Carry_FA & Sum_FA));

tenth <= TotalSum / 10;

unit  <= TotalSum mod 10;

HEX1 <= SevenSegmentDisplay(tenth);

HEX0 <= SevenSegmentDisplay(unit);

end if;

end process;

end Behavioral;

## Verilog code

module MTran_Lab6_Verilog_Adder(

input  wire [9:8] mode,

input  wire [3:0] A,

input  wire [3:0] B,

output reg  [7:0] LED,

output reg  [6:0] HEX1,

output reg  [6:0] HEX0

);

/********************Variable and Signal Declaration**************************/

wire [3:0] Sum_HA;

wire [3:0] Carry_HA;

wire [3:0] Difference_HS;

wire [3:0] Borrow_HS;

wire [3:0] Sum_FA;

reg Cin;

wire Carry_FA;

reg [4:0] TotalSum;

reg [4:0] tenth;

reg [4:0] units;

/********************End of Variable and Signal Declaration*******************/

/********************Module Instantiation*************************************/

MTran_Lab6_Verilog_HalfAdder HA (

.A(A),

.B(B),

.Sum(Sum_HA),

.Carry(Carry_HA)

);

MTran_Lab6_Verilog_HalfSubtractor HS (

.A(A),

.B(B),

.Difference(Difference_HS),

.Borrow(Borrow_HS)

);

MTran_Lab6_Verilog_FullAdder FA (

.A(A),

.B(B),

.Cin(Cin),

.Sum(Sum_FA),

.CarryOut(Carry_FA)

);

/********************End of Module Instantiation******************************/

/********************Function Declaration*************************************/

function [6:0] SevenSegmentDisplay;

input reg [4:0] Num;

begin

case(Num)

5'd0: SevenSegmentDisplay = 7'b1000000;

5'd1: SevenSegmentDisplay = 7'b1111001;

5'd2: SevenSegmentDisplay = 7'b0100100;

5'd3: SevenSegmentDisplay = 7'b0110000;

5'd4: SevenSegmentDisplay = 7'b0011001;

5'd5: SevenSegmentDisplay = 7'b0010010;

5'd6: SevenSegmentDisplay = 7'b0000010;

5'd7: SevenSegmentDisplay = 7'b1111000;

5'd8: SevenSegmentDisplay = 7'b0000000;

5'd9: SevenSegmentDisplay = 7'b0010000;

default: SevenSegmentDisplay = 7'b1111111;

endcase;

end

endfunction

/********************End of Function Declaration******************************/

// Main code

always@(mode,A,B)

begin

// Reset LED and 7 Segment

HEX1 = 7'b1111111;

HEX0 = 7'b1111111;

LED  = 8'b000000000;

// Change mode

if (mode == 2'b00)

begin

LED[3:0] = Sum_HA;

LED[7:4] = Carry_HA;

end

else if (mode == 2'b01)

begin

LED[3:0] = Difference_HS;

LED[7:4] = Borrow_HS;

end

else if (mode == 2'b10)

begin

Cin = 1'b0;

LED[3:0] = Sum_FA;

LED[7:4] = Carry_FA;

TotalSum = {Carry_FA, Sum_FA};

tenth = TotalSum / 10;

units = TotalSum % 10;

HEX1 = SevenSegmentDisplay(tenth);

HEX0 = SevenSegmentDisplay(units);

end

else if (mode == 2'b11)

begin

Cin = 1'b1;

LED[3:0] = Sum_FA;

LED[7:4] = Carry_FA;

TotalSum = {Carry_FA, Sum_FA};

tenth = TotalSum / 10;

units = TotalSum % 10;

HEX1 = SevenSegmentDisplay(tenth);

HEX0 = SevenSegmentDisplay(units);

end

end

endmodule

# Pin Planner

## Input and Output

Figure 4-1: SCEMA5F31C6N board

Assigning:

SW[0]:         SW0

SW[1]:         SW1

SW[2]:         SW2

Mode[0]:         SW7

Mode[1]:         SW8

EN:         	SW9

LED[0]:       LEDR0

LED[1]:       LEDR1

LED[2]:       LEDR2

LED[3]:       LEDR3

LED[4]:       LEDR4

LED[5]:       LEDR5

LED[6]:       LEDR6

LED[7]:       LEDR7

HEX0[0]:       HEX0[0]

HEX0[1]:       HEX0[1]

HEX0[2]:       HEX0[2]

HEX0[3]:       HEX0[3]

HEX0[4]:       HEX0[4]

HEX0[5]:       HEX0[5]

HEX0[6]:       HEX0[6]

From the DE1_SoC_User_Manual,

Figure 4-2: SW0 and SW1 Pin No

Figure 4-3: LEDR0's Pin No

Figure 4-4: HEX's Pin No

# Result

## Half Adder

With inputA = “0001” and inputB = “1001”, Sum should be “1000” and Carry is “0001”

Figure 5-1: Output for Half Adder.

## Half Subtractor

With inputA = “1000” and inputB = “1011”, Difference should be “0011” and Carry is “0011”

Figure 5-2: Output for Half Subtractor

REFERENCES