LAB NOTE

Subject: Digital Design Principles

Topic: Magnitude Comparator

Student: Minh Quan Tran

July 12th, 2024

Table of Contents

1.	Objectives	4

2.	Theory and Design	6

2.1	Theory	6

2.2	Requirement	6

2.3	Solution	6

3.	VHDL and Verilog	6

3.1	VHDL code	6

3.2	Verilog code	9

4.	Pin Planner	14

4.1	Input and Output	14

5.	Result	16

TABLE OF FIGURES

Figure 2-1: Octal Decoder's Truth Table	6

Figure 4-1: SCEMA5F31C6N board	14

Figure 4-2: SW0 and SW1 Pin No	15

Figure 4-3: LEDR0's Pin No	15

Figure 4-4: HEX's Pin No	15

Figure 5-1: When input = ‘000’.	16

Figure 5-2: When input = ‘111’	16

Figure 5-3: When input ‘101’	17

Figure 5-4: Input ‘101’ with Complement mode	17

Figure 5-5: Input ‘101’ with Multiline mode	18

Figure 5-6: Result for adding them up	18

# Objectives

- Design a Magnitude Comparator using Quartus software.

- Write a VHDL and Verilog code from the design.

- Push the code and run to the SCEMA5F31C6N board.

# Theory and Design

## Theory

Magnitude Comparator is a circuit that compare 2 inputs A and B and give out either 3 output ‘100’, ‘010’ and ‘001’ which should represent Greater Than, Equal, Less Than.

## Requirement

- Use VHDL and Verilog to implement a Magnitude Comparator and display to LED [0:2].

- Introduction to the VHDL Case statement.

- Above and beyond:  Adding mode: Clock count up from 00:00:00 to 23:59:59 with 2 speed normal speed and 100x speed.

## Solution

To design this system, first need to identify input and output:

- Input will be from SW0 to SW7.

- Output will be LED[2:0].

Then identify what the output will be, from the 2 inputs,

Figure 2-1: Magnitude Comparator’s Truth Table

# VHDL and Verilog

## VHDL code

### Top entity

library ieee;

use ieee.std_logic_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity MTran_Lab7_VHDL_MagnitudeComparator is

port

(

-- Inputs

CLOCK_50 : in  std_logic;

A        : in  std_logic_vector (3 downto 0);

B        : in  std_logic_vector (3 downto 0);

mode     : in  std_logic;

-- Outputs

LED   : out std_logic_vector (2 downto 0);

HEX0  : out std_logic_vector (6 downto 0);

HEX1  : out std_logic_vector (6 downto 0);

HEX2  : out std_logic_vector (6 downto 0);

HEX3  : out std_logic_vector (6 downto 0);

HEX4  : out std_logic_vector (6 downto 0);

HEX5  : out std_logic_vector (6 downto 0)

);

end entity;

architecture Behavioral of MTran_Lab7_VHDL_MagnitudeComparator is

-- Module declaration

component MTran_Lab7_VHDL_Timer is

port

(

-- Inputs

Clk     : in std_logic;

Enable  : in std_logic;

-- Outputs

Seconds : inout integer;

Minutes : inout integer;

Hours   : inout integer

);

end component;

-- Singal declaration

signal intA        : integer := 0;

signal intB        : integer := 0;

signal Enable      : std_logic;

signal Seconds     : integer := 0;

signal Minutes     : integer := 0;

signal Hours       : integer := 0;

-- Function

function SevenSegmentDisplay(Number : integer) return std_logic_vector is

begin

case (Number) is

when 0       => return "1000000";

when 1        => return "1111001";

when 2        => return "0100100";

when 3         => return "0110000";

when 4        => return "0011001";

when 5         => return "0010010";

when 6        => return "0000010";

when 7      => return "1111000";

when 8      => return "0000000";

when 9      => return "0010000";

when others => return "1111111";

end case;

end SevenSegmentDisplay;

-- Main code

begin

-- Module instatiate

module_timer: MTran_Lab7_VHDL_Timer

port map

(

Clk       => CLOCK_50,

Enable  => Enable,

Seconds => Seconds,

Minutes => Minutes,

Hours   => Hours

);

-- Process

process(CLOCK_50, Seconds, Minutes, Hours, A, B) is

begin

if mode = '0' then

-- Disable Timer

Enable <= '0';

-- Reset HEX2

HEX2 <= "1111111";

-- Convert A and B from std_logic to integer

intA <= to_integer(unsigned(A));

intB <= to_integer(unsigned(B));

-- Display A and B in decimal

-- A

HEX0 <= SevenSegmentDisplay(intA mod 10);

HEX1 <= SevenSegmentDisplay(intA / 10);

-- B

HEX4 <= SevenSegmentDisplay(intB mod 10);

HEX5 <= SevenSegmentDisplay(intB / 10);

-- Compare A and B

if intA > intB then

LED <= "100";

HEX3 <= "0100111";

elsif intA = intB then

LED <= "010";

HEX3 <= "0110111";

else

LED <= "001";

HEX3 <= "0110011";

end if;

else

Enable <= '1';

LED <= "000";

-- Display Second

HEX0 <= SevenSegmentDisplay(Seconds mod 10);

HEX1 <= SevenSegmentDisplay(Seconds / 10);

-- Display Minute

HEX2 <= SevenSegmentDisplay(Minutes mod 10);

HEX3 <= SevenSegmentDisplay(Minutes / 10);

-- Display Hour

HEX4 <= SevenSegmentDisplay(Hours mod 10);

HEX5 <= SevenSegmentDisplay(Hours / 10);

end if;

end process;

end architecture;

### Component Timer

library ieee;

use ieee.std_logic_1164.ALL;

entity MTran_Lab7_VHDL_Timer is

port

(

-- Inputs

Clk     : in    std_logic;

Enable  : in    std_logic;

-- Outputs

Seconds : inout integer := 0;

Minutes : inout integer := 0;

Hours   : inout integer := 0

);

end entity;

architecture RTL of MTran_Lab7_VHDL_Timer is

-- Signal Declaration

signal Ticks                : integer := 0;

-- Procedure

procedure Increment( signal   Counter   : inout integer;

constant MaxValue  : in    integer;

variable ResetFlag : out   boolean) is

begin

if Counter = MaxValue - 1 then

-- Reset Counter

Counter  <= 0;

ResetFlag := True;

else

Counter <= Counter + 1;

ResetFlag := False;

end if;

end procedure;

begin

process(Clk, Enable) is

-- Variable Declaring

variable SecondsFlag      : boolean := false;

variable MinutesFlag      : boolean := false;

variable HoursFlag        : boolean := false;

constant MaxSeconds       : integer := 60;

constant MaxMinutes       : integer := 60;

constant MaxHours         : integer := 24;

constant ClockFrequency   : integer := 50000000;

-- Main

begin

if Enable = '1' then

if rising_edge(Clk) then

-- Increment Ticks

Increment(Ticks, ClockFrequency, SecondsFlag);

-- Increment Seconds whenever Ticks reach Clock Frequency

if SecondsFlag then

Increment(Seconds, MaxSeconds, MinutesFlag);

end if;

-- Increment Minute whenever 60 seconds

if SecondsFlag and MinutesFlag then

Increment(Minutes, MaxMinutes, HoursFlag);

end if;

-- Increment Hours whenever 60 minutes

if SecondsFlag and MinutesFlag and HoursFlag then

Increment(Hours, MaxHours, HoursFlag);

end if;

end if;

end if;

end process;

end architecture;

## Verilog code

### Top Module

module MTran_Lab7_Verilog_MagnitudeComparator

(

input wire CLOCK_50,

input wire [3:0] A,

input wire [3:0] B,

input mode,

input speed,

output reg [2:0] LED,

output reg [6:0] HEX0,

output reg [6:0] HEX1,

output reg [6:0] HEX2,

output reg [6:0] HEX3,

output reg [6:0] HEX4,

output reg [6:0] HEX5

);

// Signals declaration

reg [31:0] intA;

reg [31:0] intB;

reg Enable;

wire [31:0] Seconds1;

wire [31:0] Minutes1;

wire [31:0] Hours1;

wire [31:0] Seconds2;

wire [31:0] Minutes2;

wire [31:0] Hours2;

// Module Instantiation

MTran_Lab7_Verilog_Timer

#(

.MAXSECONDS(60),

.MAXMINUTES(60),

.MAXHOURS(24),

.CLKFREQUENCY(500000) // 500kHz => Clock 10 times faster

)

Tim1

(

.Clk(CLOCK_50),

.Enable(Enable),

.Seconds(Seconds1),

.Minutes(Minutes1),

.Hours(Hours1)

);

MTran_Lab7_Verilog_Timer

#(

.MAXSECONDS(60),

.MAXMINUTES(60),

.MAXHOURS(24),

.CLKFREQUENCY(50000000) // 50 MHz => Normal Clk

)

Tim2

(

.Clk(CLOCK_50),

.Enable(Enable),

.Seconds(Seconds2),

.Minutes(Minutes2),

.Hours(Hours2)

);

// Function for Seven Segment Display

function [6:0] SevenSegmentDisplay;

input [3:0] Number;

begin

case (Number)

4'h0: SevenSegmentDisplay = 7'b1000000;

4'h1: SevenSegmentDisplay = 7'b1111001;

4'h2: SevenSegmentDisplay = 7'b0100100;

4'h3: SevenSegmentDisplay = 7'b0110000;

4'h4: SevenSegmentDisplay = 7'b0011001;

4'h5: SevenSegmentDisplay = 7'b0010010;

4'h6: SevenSegmentDisplay = 7'b0000010;

4'h7: SevenSegmentDisplay = 7'b1111000;

4'h8: SevenSegmentDisplay = 7'b0000000;

4'h9: SevenSegmentDisplay = 7'b0010000;

default: SevenSegmentDisplay = 7'b1111111;

endcase

end

endfunction

always @(posedge CLOCK_50)

begin

if (mode == 1'b0)

begin

// Disable Timer

Enable = 1'b0;

// Reset HEX2

HEX2 = 7'b1111111;

// Convert A and B from std_logic to integer

intA = A;

intB = B;

// Display A and B in decimal

// A

HEX0 = SevenSegmentDisplay(intA % 10);

HEX1 = SevenSegmentDisplay(intA / 10);

// B

HEX4 = SevenSegmentDisplay(intB % 10);

HEX5 = SevenSegmentDisplay(intB / 10);

// Compare A and B

if (intA > intB)

begin

LED  = 3'b100;

HEX3 = 7'b0100111;

end

else if (intA == intB)

begin

LED  = 3'b010;

HEX3 = 7'b0110111;

end

else

begin

LED  = 3'b001;

HEX3 = 7'b0110011;

end

end

else

begin

Enable = 1'b1;

LED    = 3'b000;

if (speed == 1'b0) //normal mode

begin

// Display Second

HEX0  = SevenSegmentDisplay(Seconds2 % 10);

HEX1  = SevenSegmentDisplay(Seconds2 / 10);

// Display Minute

HEX2  = SevenSegmentDisplay(Minutes2 % 10);

HEX3  = SevenSegmentDisplay(Minutes2 / 10);

// Display Hour

HEX4  = SevenSegmentDisplay(Hours2 % 10);

HEX5  = SevenSegmentDisplay(Hours2 / 10);

end

else if (speed == 1'b1)

begin

// Display Second

HEX0  = SevenSegmentDisplay(Seconds1 % 10);

HEX1  = SevenSegmentDisplay(Seconds1 / 10);

// Display Minute

HEX2  = SevenSegmentDisplay(Minutes1 % 10);

HEX3  = SevenSegmentDisplay(Minutes1 / 10);

// Display Hour

HEX4  = SevenSegmentDisplay(Hours1 % 10);

HEX5  = SevenSegmentDisplay(Hours1 / 10);

end

end

end

endmodule

### Module Timer

module MTran_Lab7_Verilog_Timer

#(

parameter MAXSECONDS      = 60,

parameter MAXMINUTES      = 60,

parameter MAXHOURS        = 24,

parameter CLKFREQUENCY    = 50000000

)

(

input wire Clk,

input wire Enable,

inout reg [31:0] Seconds,

inout reg [31:0] Minutes,

inout reg [31:0] Hours

);

// Signal Declaration

reg [31:0] Ticks = 0;

// Main Logic

always @(posedge Clk)

begin

if (Enable)

begin

// Increment Ticks

Ticks = Ticks + 1;

if (Ticks == CLKFREQUENCY - 1)

begin

Ticks   = 0;

Seconds = Seconds + 1;

end

if (Seconds == MAXSECONDS)

begin

Seconds = 0;

Minutes = Minutes + 1;

end

if (Minutes == MAXMINUTES)

begin

Minutes = 0;

Hours   = Hours + 1;

end

if (Hours == MAXHOURS)

begin

Hours   = 0;

end

end

end

endmodule

# Pin Planner

## Input and Output

Figure 4-1: SCEMA5F31C6N board

Assigning:

SW[0]:         SW0

SW[1]:         SW1

SW[2]:         SW2

Mode[0]:         SW7

Mode[1]:         SW8

EN:         	SW9

LED[0]:       LEDR0

LED[1]:       LEDR1

LED[2]:       LEDR2

LED[3]:       LEDR3

LED[4]:       LEDR4

LED[5]:       LEDR5

LED[6]:       LEDR6

LED[7]:       LEDR7

HEX0[0]:       HEX0[0]

HEX0[1]:       HEX0[1]

HEX0[2]:       HEX0[2]

HEX0[3]:       HEX0[3]

HEX0[4]:       HEX0[4]

HEX0[5]:       HEX0[5]

HEX0[6]:       HEX0[6]

From the DE1_SoC_User_Manual,

Figure 4-2: SW0 and SW1 Pin No

Figure 4-3: LEDR0's Pin No

Figure 4-4: HEX's Pin No

# Result

Figure 5-1: When input = ‘000’.

SW2 = 0, SW1 = 0, SW0 = 0 which should be 0, LED0 asserted and the 7 segment LED displaying 0.

Figure 5-2: When input = ‘111’

SW2 = 1, SW1 = 1, SW0 = 1 which should be 7, LED7 asserted and the 7 segment LED displaying 7.

Figure 5-3: When input ‘101’

SW2 = 1, SW1 = 0, SW0 = 1 which should be 5, LED5 asserted and the 7 segment LED displaying 5.

Different mode:

- Complement mode:

Figure 5-4: Input ‘101’ with Complement mode

In this mode, rather than LED5 assert, all other LED except LED5 asserted. To activate this mode SW [8:7] = ‘01’.

- Multiline Mode:

Figure 5-5: Input ‘101’ with Multiline mode

In this mode, when input is ‘101’ which is 5, LED0 to LED5 lit up. To activate this mode SW [8:7] = ‘10’.

- Complement Multiline Mode:

Figure 5-6: Result for adding them up

In this mode, it take the output of Multiline Mode and complemented it. To activate this mode SW [8:7] = ‘11’

REFERENCES