LAB NOTE

Subject: Digital Design Principles

Topic: Shift Register

Student: Minh Quan Tran

July 26th, 2024

Table of Contents

1.	Objectives	4

2.	Theory and Design	6

2.1	Theory	6

2.1.1	Ring Counter	6

2.1.2	Johnson Counter	6

2.2	Requirement	6

2.3	Solution	6

3.	VHDL and Verilog	6

3.1	VHDL code	6

3.1.1	Top entity	6

3.1.2	Component Ring Counter	8

3.1.3	Component Ring Counter	9

3.2	Verilog code	10

3.2.1	Top Module	10

3.2.2	Module Timer	13

4.	Pin Planner	15

4.1	Input and Output	15

5.	Result	17

TABLE OF FIGURES

Figure 2-1: Counters’s Truth Table	6

Figure 4-1: SCEMA5F31C6N board	15

Figure 4-2: SW0 and SW1 Pin No	16

Figure 4-3: LEDR0's Pin No	16

Figure 4-4: HEX's Pin No	16

Figure 5-1: When input = ‘000’.	17

Figure 5-2: When input = ‘111’	17

Figure 5-3: When input ‘101’	18

Figure 5-4: Input ‘101’ with Complement mode	18

Figure 5-5: Input ‘101’ with Multiline mode	19

Figure 5-6: Result for adding them up	19

# Objectives

- Design a Counters using Quartus software.

- Write a VHDL and Verilog code from the design.

- Push the code and run to the SCEMA5F31C6N board.

# Theory and Design

## Theory

A shift register is a digital circuit used for storing and transferring data, consisting of a series of flip-flops connected in a chain. Data is shifted through these flip-flops sequentially, either left or right, with each clock pulse. There are various types of shift registers, including Serial-In Serial-Out (SISO), Serial-In Parallel-Out (SIPO), Parallel-In Serial-Out (PISO), and Parallel-In Parallel-Out (PIPO), each serving different data input and output requirements. Shift registers are crucial in digital systems for tasks such as data storage, data transfer, and data conversion between serial and parallel formats.

## Requirement

- Use VHDL and Verilog to implement the shift and display to LED [1:4].

- Introduction to the VHDL Case statement.

- Above and beyond:  Adding Complement mode

## Solution

To design this system, first need to identify input and output:

- Input will be from BTN0 to BTN3

  - BTN0 : CLK

  - BTN1 : Speed

  - BTN3 : Reset

- Output will be LED[4:1].

Then identify what the output will be, from the 2 inputs,

Figure 2-1: Shift Register’s Truth Table

# VHDL and Verilog

## VHDL code

### Top entity

library ieee;

use ieee.std_logic_1164.all;

entity MTran_Lab9_VHDL_ShiftRegister is

port

(

-- Inputs

BTN   : in std_logic;

Speed : in std_logic;

Reset : in std_logic;

SW    : in std_logic_vector (7 downto 0);

-- Outputs

LED   : out std_logic_vector (3 downto 0);

HEX0  : out std_logic_vector (6 downto 0)

);

end entity;

architecture behavioral of MTran_Lab9_VHDL_ShiftRegister is

-- Signal Declaration

signal Counter : integer                       := 1;

signal Output  : std_logic_vector (3 downto 0) := "0000";

signal Sel     : std_logic_Vector (1 downto 0);

-- Function declaration

function SevenSegmentDisplay (Number : integer) return std_logic_vector is

begin

case (Number) is

when 0	   => return "1000000";

when 1		=> return "1111001";

when 2		=> return "0100100";

when 3 		=> return "0110000";

when 4		=> return "0011001";

when 5 		=> return "0010010";

when 6		=> return "0000010";

when 7      => return "1111000";

when 8      => return "0000000";

when 9      => return "0010000";

when others => return "1111111";

end case;

end SevenSegmentDisplay;

begin

-- Process

-- Check Sel

process(SW(6),SW(7)) is

begin

Sel <= SW(7) & SW(6);

end process;

-- Check Speed

process(Speed) is

begin

if Speed ='0' then

if Counter < 4 then

Counter <= Counter * 2;

else

Counter <= 1;

end if;

end if;

end process;

-- Main code (Shift)

process(Sel, SW, BTN, Reset) is

begin

-- Reset BTN

if Reset = '0' then

Output  <= "0000";

elsif rising_edge(BTN) then

if Counter = 1 then

case (Sel) is

when "00" => NULL;

when "01" => Output <= Output(2 downto 0) & SW(0);

when "10" => Output <= SW(5) & Output(3 downto 1);

when "11" => Output <= SW(4 downto 1);

end case;

elsif Counter = 2 then

case (Sel) is

when "00" => NULL;

when "01" => Output <= Output(1 downto 0) & SW(0) & SW(0);

when "10" => Output <= SW(5) & SW(5) & Output(3 downto 2);

when "11" => Output <= SW(4 downto 1);

end case;

elsif Counter = 4 then

case (Sel) is

when "00" => NULL;

when "01" => Output <= SW(0) & SW(0) & SW(0) & SW(0);

when "10" => Output <= SW(5) & SW(5) & SW(5) & SW(5);

when "11" => Output <= SW(4 downto 1);

end case;

end if;

end if;

end process;

-- Output

process(Output, Counter)

begin

LED  <= Output;

HEX0 <= SevenSegmentDisplay(Counter);

end process;

end Behavioral;

## Verilog code

module MTran_Lab9_Verilog_ShiftRegister

(

input wire BTN,

input wire Speed,

input wire Reset,

input wire [7:0] SW,

output reg [3:0] LED,

output reg [6:0] HEX0

);

// Signal Declarations

reg [3:0] Output;

reg [1:0] Sel;

integer Counter = 1;

// Seven Segment Display function

function [6:0] SevenSegmentDisplay;

input integer Number;

begin

case (Number)

0: SevenSegmentDisplay = 7'b1000000;

1: SevenSegmentDisplay = 7'b1111001;

2: SevenSegmentDisplay = 7'b0100100;

3: SevenSegmentDisplay = 7'b0110000;

4: SevenSegmentDisplay = 7'b0011001;

5: SevenSegmentDisplay = 7'b0010010;

6: SevenSegmentDisplay = 7'b0000010;

7: SevenSegmentDisplay = 7'b1111000;

8: SevenSegmentDisplay = 7'b0000000;

9: SevenSegmentDisplay = 7'b0010000;

default: SevenSegmentDisplay = 7'b1111111;

endcase

end

endfunction

// Check Sel

always @(*)

begin

Sel = {SW[7], SW[6]};

end

// Check Speed

always @(negedge Speed)

begin

if (Speed == 0)

begin

if (Counter < 4)

Counter = Counter * 2;

else

Counter = 1;

end

end

// Main code (Shift)

always @(negedge BTN or negedge Reset)

begin

if (Reset == 0)

Output <= 4'b0000;

else if (BTN == 0)

begin

if (Counter == 1)

begin

case (Sel)

2'b00: Output = Output;

2'b01: Output = {Output[2:0], SW[0]};

2'b10: Output = {SW[5], Output[3:1]};

2'b11: Output = SW[4:1];

endcase

end

else if (Counter == 2)

begin

case (Sel)

2'b00: Output = Output;

2'b01: Output = {Output[1:0], SW[0], SW[0]};

2'b10: Output = {SW[5], SW[5], Output[3:2]};

2'b11: Output = SW[4:1];

endcase

end

else if (Counter == 4)

begin

case (Sel)

2'b00: Output = Output;

2'b01: Output = {SW[0], SW[0], SW[0], SW[0]};

2'b10: Output = {SW[5], SW[5], SW[5], SW[5]};

2'b11: Output = SW[4:1];

endcase

end

end

end

// Output

always @(*)

begin

LED  = Output;

HEX0 = SevenSegmentDisplay(Counter);

end

endmodule

# Pin Planner

## Input and Output

Figure 4-1: SCEMA5F31C6N board

Assigning:

SW[0]:         SW0

SW[1]:         SW1

SW[2]:         SW2

Mode[0]:         SW7

Mode[1]:         SW8

EN:         	SW9

LED[0]:       LEDR0

LED[1]:       LEDR1

LED[2]:       LEDR2

LED[3]:       LEDR3

LED[4]:       LEDR4

LED[5]:       LEDR5

LED[6]:       LEDR6

LED[7]:       LEDR7

HEX0[0]:       HEX0[0]

HEX0[1]:       HEX0[1]

HEX0[2]:       HEX0[2]

HEX0[3]:       HEX0[3]

HEX0[4]:       HEX0[4]

HEX0[5]:       HEX0[5]

HEX0[6]:       HEX0[6]

From the DE1_SoC_User_Manual,

Figure 4-2: SW0 and SW1 Pin No

Figure 4-3: LEDR0's Pin No

Figure 4-4: HEX's Pin No

# Result

Figure 5-1: When input = ‘000’.

SW2 = 0, SW1 = 0, SW0 = 0 which should be 0, LED0 asserted and the 7 segment LED displaying 0.

Figure 5-2: When input = ‘111’

SW2 = 1, SW1 = 1, SW0 = 1 which should be 7, LED7 asserted and the 7 segment LED displaying 7.

Figure 5-3: When input ‘101’

SW2 = 1, SW1 = 0, SW0 = 1 which should be 5, LED5 asserted and the 7 segment LED displaying 5.

Different mode:

- Complement mode:

Figure 5-4: Input ‘101’ with Complement mode

In this mode, rather than LED5 assert, all other LED except LED5 asserted. To activate this mode SW [8:7] = ‘01’.

- Multiline Mode:

Figure 5-5: Input ‘101’ with Multiline mode

In this mode, when input is ‘101’ which is 5, LED0 to LED5 lit up. To activate this mode SW [8:7] = ‘10’.

- Complement Multiline Mode:

Figure 5-6: Result for adding them up

In this mode, it take the output of Multiline Mode and complemented it. To activate this mode SW [8:7] = ‘11’

REFERENCES