LAB NOTE

Subject: Electronic Design Principles

Topic: Scale Adder

Student: Minh Quan Tran

June 7th , 2024

Table of Contents

1.	Objectives	4

2.	Theory and Calculation	5

2.1	Theory	5

2.2	Design and Calculation	5

3.	Design and Result	6

3.1	Multisim’s design.	6

3.2	Breadboard’s design	6

3.3	Result	7

3.3.1	DC power	7

3.3.2	AC power	8

4.	CONCLUSION	9

TABLE OF FIGURES

Figure 2-1: Scale Adder's design	5

Figure 2-2: LM348N's datasheet	6

Figure 3-1: Multisim's design	6

Figure 3-2: Breadboard’s design	6

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

TABLE’S OF TABLE

Table 3-1: DC power for Scale Adder	7

Table 3-4: AC power for Scale Adder	8

# Objectives

- Build a scaled adder that one inverting input with R1 will be amplified by 10 and the other R2 by 5.

- Use 10 kOhm resister for one of the input resisters, R1 and calculate Rf. Use the same Rf to calculate R2.

- Show your design in using Multisim in your pre-lab.

- Prepare a table for measurement in advance to prove that your circuit work by comparing your calculated values with the actual values.

# Theory and Calculation

## Theory

- A scaling adder has two or more inputs with each input having a different gain. The output represents the negative scaled sum of the inputs.

## Design and Calculation

Figure 2-1: Scale Adder's design

For this design the following equipment are use:

- Resistor 10kΩ, Resistor 22kΩ and Resistor 100kΩ

- IC LM348N

- Function Generator

- Power Supply

Formula to calculate gain:

Ai= -RfRi

Formula to calculate Vout:

Vout= i =1∞Ai×Vini

- Calculation for Rf:

The requirement stated that it need the gain of 10 for input 1 and gain 2 for input 2, with R1 = 10kΩ:

A1= -RfR1

- 10= -Rf10kΩ

Rf=100kΩ

- Calculation for R2:

A2= -RfR2

-5= -100kΩR1

R2=20kΩ

Because the kit does not have a resistor with value 20kΩ, so instead will use 22kΩ resistor.

- Power supply:

Figure 2-2: LM348N's datasheet

From the datasheet, it said that it recommended that the power supply of Vcc+ should be in range from 4V to 18V, and for Vcc- should be from -4 to -18.

# Design and Result

For this assignment, 6 cases will be tested, 3 will be with DC power and 3 will be from AC power.

## Multisim’s design.

Figure 3-1: Multisim's design

## Breadboard’s design

5

Figure 3-2: Breadboard’s design

## Result

### DC power

| Input | Output |  |  |  |
| --- | --- | --- | --- | --- |
|  | Theory | Multisim |  | Breadboard |
| -0.28 | 4.2 |  |  |  |
| -0.4 | 6 |  |  |  |
| -0.64 | 9.6 |  |  |  |

Table 3-1: DC power for Scale Adder

### AC power

- Inverting

| Input (Vpp) | Output |  |  |
| --- | --- | --- | --- |
|  | Theory | Multisim | Breadboard |
| 0.56 | 7.4 |  |  |
| 1.12 | 16.8 |  |  |
| 0.62 | 9.3 |  |  |

Table 3-2: AC power for Scale Adder

# CONCLUSION

From the result of all cases:

- All cases have Theory’s result, Multisim’s result and Breadboard’s result nearly the same.

REFERENCES